| Problem Chosen B | 2026 MCM/ICM Summary Sheet | Team Control Number 2601727 |
| --- | --- | --- |

Balancing Velocity, Economy and Ecology: A Robust Hybrid Logistics Framework for Sustainable Lunar Colonization

Summary

The establishment of a self-sustaining lunar colony imposes unprecedented logistical challenges characterized by massive volumes (100 million tons), extreme costs, and stochastic risks. This paper develops LUNALINK, a comprehensive systems engineering framework designed to optimize the Earth-Moon logistics network by balancing the immediacy of chemical rockets with the long-term sustainability of space elevators.

For Phase I: Strategic Logistics Planning, we developed a Dynamic Logistics Optimization Model using Mixed-Integer Programming (MIP) to determine the optimal transport architecture. Integrating Wright’s Law to simulate technological learning curves, we analyzed the cost-time trade-off. Our results reject single-mode strategies, favoring a Hybrid Strategy where rockets provide early-stage responsiveness while Space Elevators handle bulk transport. In a deterministic environment, this approach minimizes the weighted objective of total cost and construction duration.

For Phase II: Resilience & Risk Quantification, we transitioned from a static optimum to a stochastic reality. Addressing the 0% success rate of deterministic plans under Monte Carlo stress testing, we introduced a "Dual-Buffer" Robust Optimization Strategy, incorporating demand-side safety stocks and supply-side capacity discount factors. While this resilience strategy increases the budget to $86.3 Trillion and extends the timeline to 147 years, it restores the mission success rate to 100%, effectively quantifying the "insurance premium" of reliability.

For Phase III: Operational Metabolism, we constructed a Metabolic Supply Chain Model based on Inventory Control Theory to sustain a 100,000-person colony. Analysis confirms the space elevator as the sole financially viable artery, reducing marginal transport costs by 99.9% compared to rocketry. Our proposed "Weekly Pulse" Schedule ensures water inventory remains above safety thresholds even during 45-day elevator outages.

For Phase IV: Environmental Evaluation, we internalized negative externalities such as atmospheric pollution and space debris into a Multi-Objective Optimization Model. By defining an Environmental Impact Index (EII), we identified a Pareto-optimal decision interval for the environmental weight parameter . This balance suppresses environmental damage by over 60% while maintaining a feasible construction schedule, advocating for a shift from pure cost minimization to Sustainable Interstellar Governance.

In conclusion, LUNALINK validates that sustainable interstellar expansion requires shifting from pure cost minimization to robust governance. Validated by Morris Sensitivity Analysis, our framework provides the Moon Colony Management (MCM) Agency with a scientifically grounded, risk-resistant roadmap.

Key words: Earth-Moon Logistics; Hybrid Transportation; Robust Optimization; Monte Carlo Simulation; Space Elevator; Sustainable Governance

Contents

1 Introduction	3

1.1	Background	3

1.2	Problem Restatement	3

1.3	Our works	4

2 Assumption & Justification	5

3 Notations	6

4 Model Preparation	6

4.1	Data Overview	6

4.2	Data Collections	7

4.3	Vital Parameter Estimations	7

5 Model Design	9

5.1	Phase I: Deterministic Logistics Model Design & Economic Assessment	9

5.1.1  Definition of Decision Variables & Construction of Multi-Objective Function	9

5.1.2  Scenario-based Transportation Planning and Modeling	9

5.1.3  Basical Model Result	11

5.2	Phase II: Stochastic Resilience & Adaptive Strategy	12

5.2.1  Mathematical Representation of Risk Features	12

5.2.2  Monte Carlo Stress Testing and Model Vulnerability Analysis	13

5.2.3  Dual-Buffer Robust Optimization Strategy	14

5.2.4  Robustness Validation and Risk Premium	14

5.2.5  Adaptive Substitution Mechanism under Extreme Scenarios	15

5.3	Phase III: Long-term Metabolic Supply Chain & Inventory Management	16

5.3.1  Analysis of Annual Total Water Demand for a Lunar Colony	16

5.3.2  Supply Channel Selection and Economic Evaluation	17

5.3.3  Inventory Dynamics & Schedule Optimization	17

5.3.4  Optimized Resupply Schedule and Strategic Analysis	18

5.4	Phase IV: Environmental Exteriority & Green Evaluation	19

5.4.1  3D Multi-objective Optimization Strategy	19

5.4.2  Analysis of Environmental Trade-offs	20

6 Sensitivity Analysis	20

6.1 Global Parameter Screening: The Morris Method Approach	20

6.2 Quantitative Local Sensitivity Analysis	21

7 Strengths and Weaknesses	22

7.1 Strengths	22

7.2 Weaknesses	23

8 Conclusion	23

References	25

1 Introduction

Background

The transition from extraterrestrial exploration to permanent colonization represents the next "Great Filter" for human civilization. As we approach the mid-21st century, the ambition to establish a self-sustaining lunar colony for 100,000 inhabitants by 2050 has moved from science fiction to engineering reality.

However, it presents an unprecedented logistical challenge: transporting 100 million metric tons of infrastructure materials. This endeavor exposes a fundamental trade-off between traditional chemical rocketry—characterized by high speed but prohibitive costs and emissions—and the proposed Space Elevator System, which offers sustainable, low-cost transit but faces capacity constraints and significant upfront deployment risks.

The Moon Colony Management (MCM) Agency must navigate this complex landscape to optimize construction schedules and budgets. The core challenge lies in balancing the immediate efficiency of rockets against the long-term sustainability of space elevators, while simultaneously ensuring system resilience against catastrophic failures and minimizing the environmental footprint on Earth.

Figure 1：The Falcons, The Moon & The elevator

Problem Restatement

To assist the MCM Agency in formulating a scientifically grounded roadmap, we model the Earth-Moon logistics network as a complex, dynamic system. Our research objectives are decomposed into four distinct but interconnected tasks:

- Task 1: Strategic Logistics Planning (Phase I). Develop a multi-objective optimization model to determine the optimal transport architecture (Elevator, Rocket, or Hybrid). The objective is to construct a Pareto frontier that minimizes both Total Cost ($) and Construction Timeline (Years).

- Task 2: Resilience & Risk Quantification (Phase II). Assess system vulnerability to stochastic disruptions, specifically rocket launch failures and elevator tether swaying. We aim to design a "Dual-Buffer" strategy to ensure mission success without unmanageable budget overruns.

- Task 3: Operational Metabolism (Phase III). Upon colony completion, focus shifts to sustaining the population. We model the colony's water cycle and biological lock-in to determine the most economically viable "Weekly Pulse" replenishment schedule.

- Task 4: Environmental Evaluation (Phase IV). Quantify negative externalities (e.g., atmospheric pollution, space debris) and propose optimization strategies to transition the project from pure engineering efficiency to Sustainable Interstellar Governance.

Our works

To intuitively reflect our research process, a comprehensive modeling framework is designed as shown in Figure 2. Our work is structured into four core modules:

- Fhase I: Dynamic Logistics Optimization. Solving the 100M-ton transportation schedule using a Mixed-Integer Programming (MIP) approach.

- Fhase II: System Reliability Model. Assessing the impact of equipment failures and designing resilient response strategies.

- Fhase III: Resource Supply Model. Calculating long-term water demand and sustainable supply cycles for the lunar colony.

- Fhase IV: Environmental Exteriority Evaluation. Quantifying the environmental footprint of logistics activities based on Life Cycle Assessment (LCA).

Figure 2：The Framework of Our Work

2 Assumption & Justification

- Assumption 1: The model utilizes a discrete time horizon with annual accounting periods.

Justification: Long-term infrastructure projects (Capital Budgeting) are typically evaluated using annual Discounted Cash Flow models, making annual discretization consistent with standard financial reporting. Furthermore, converting continuous dynamics into a discrete-time system reduces computational complexity for century-long planning while retaining the ability to capture macroscopic strategic trends.

- Assumption 2: Operations follow techno-economic duality and exhibit learning effects.

Justification: Based on Wright’s Law, high-tech manufacturing benefits from "learning by doing," where unit costs decrease as cumulative volume increases. Distinguishing between infrastructure (Fixed Assets) and fuel/transport (Variable Costs) aligns with Cost-Volume-Profit analysis. Assuming linear variable costs allows us to focus on long-term trends rather than short-term step-cost fluctuations.

- Assumption 3: Annual Supply chain channel selection is driven by pure economic rationality.

Justification: Under the Rational Choice Theory, decision-makers are assumed to be profit-maximizers who select transport modes solely to minimize Annual Operating Expenses. This creates a "Dual Sourcing" strategy common in supply chain management, utilizing the space elevator as the "efficient source" for stability and rockets as the "responsive source" for resilience.

- Assumption 4: Inventory dynamics and supply disruption risks are deterministic.

Justification: Adopting a "Certainty Equivalent" approach avoids the computational burden of stochastic processes in a long-term strategic model. Assuming constant demand and predictable disruption durations (e.g., tether instability) aligns with the premises of the Economic Order Quantity model, allowing for the derivation of clear, optimal safety stock boundaries.

- Assumption 5: Multi-objective optimization relies on linear weighting and Pareto trade-offs.

Justification: The Weighted Sum Method is a standard approach in Multi-Attribute Utility Theory, implying that trade-offs between objectives (e.g., cost vs. environment) are compensable. This effectively constructs a Social Welfare Function where weight adjustments represent the changing "shadow prices" policy-makers assign to environmental benefits versus financial costs.

- Assumption 6: Decision standards are constrained by a bounded strategic time horizon.

Justification: Behavioral economics suggests that intertemporal choices are subject to heavy discounting; projects exceeding a specific duration suffer from diminishing strategic value regardless of cost efficiency. This constraint aligns mathematical optimization with the managerial concept of "Strategic Windows," restricting solutions to a socially and politically feasible timeframe.

3 Notations

The key mathematical notations used in this paper are listed in Table 1.

Table 1: Major notations

| Notation | Definition | Unit |
| --- | --- | --- |
|  | Total demand of material quality | metric-ton |
|  | Unit cost of space elevator from ground to summit anchor | USD |
|  | Height of space elevator | km |
|  | Payload capacity per rocket at ground launch sites | metric-ton |
|  | Unit transport cost per effective payload from ground | USD/kg |
|  | Payload factor from apex anchor to Moon-launch rocket | % |
|  | Unit transport cost per effective payload from apex anchors | USD/kg |
|  | Number of rockets launched per year from ground sites | metric-ton |
|  | Maximum annual launches at ground sites in 2050 | flts/year |
|  | Space elevator transportation capacity limit | metric-ton |
|  | Number of Galactic Harbours | count |
|  | Number of ground launch sites | count |
|  | Total construction time | year |
|  | Total cost Ratio of construction time to cost in the objective function Spacecraft/fuel optimization ratio Rocket failure probability Space elevator failure/ downtime probability | USD % % % % |

4 Model Preparation

Data Overview

Since the problem statement does not provide specific constants for all necessary physical and economic variables, we collected external data and established reasonable assumptions to define the model's parameters. We focused on extrapolating current technological trends to the year 2050. The derivation and definition of these key constants are detailed below.

Data Collections

| Data | Description |
| --- | --- |
| Falcon Heavy basic parameters Rocket launch cost Per capita municipal water uses per year Population of each country Per capita required arable land | https://www.spacex.com/vehicles/falcon-heavy. https://ourworldindata.org/grapher. https://data.apps.fao.org. https://data.worldbank.org. https://lsda.jsc.nasa.gov. |

Table 2: Data and Database Websites

Vital Parameter Estimations

We define  as the effective payload capacity of a rocket launched from a ground-based site. Assuming a baseline capacity of 150 metric tons at the equator (as defined in the problem), we model the impact of latitude while ignoring transient factors like weather. We adopt a linearized reduction model where payload capacity decreases by 0.5% for every degree of latitude deviation from the equator:

Figure 3: Estimated payload and rocket costs for each launch site

To estimate the unit cost of transporting mass from Earth to the Moon via traditional rockets, we utilized 2018 data for the Falcon Heavy launch vehicle [1][2]. However, lacking official data for direct Lunar Transfer Orbit missions, we applied a linear interpolation model based on Payload-to-Energy ratios from Low Earth Orbit and Geostationary Transfer Orbit data.

We further accounted for the additional energy required for a soft lunar landing. Using the Tsiolkovsky rocket equation with a correction parameter, and assuming a yearly technological cost reduction of 2% (cost becomes 98% of the previous year).

In this way, we estimate 2050-unit cost as: .

We estimate based on the specific orbital energy difference () between the surface and the Apex Anchor. Assuming an energy conversion efficiency  and electricity price , the cost is derived as follows:

To estimate the unit cost of transporting mass from Apex Anchor to the Moon via rockets, we assume that the transport cost is derived primarily from the energy input required.

Regarding launches from the Apex Anchor, we acknowledge that the high tangential velocity at the anchor point creates a complex orbital transfer scenario. However, for the 2050 timeframe, we assume mature orbital transfer technology allows for efficient deceleration. Considering this, we introduce a sensitivity parameter , representing the fuel consumption ratio of an Apex-launch versus a traditional Earth-launch. This allows us to express the Apex launch cost and payload ratio as functions below:

Using Carbon Dioxide Equivalent as a unified metric and assuming Metha lox propulsion, we define a baseline constant to measure the environmental impact of a single launch event. This index consists of four dimensions:

- Atmospheric Chemical Pollution: This factor accounts for the direct emissions from methane combustion and the high-altitude Global Warming Potential (GWP) of stratospheric Black Carbon (BC), calculated as:

.

- Manufacturing & Resource Consumption: This encompasses the embodied carbon from fuel production electricity (approx. ) and the scarcity shadow price of non-renewable Helium, resulting in a total resource cost of:

.

- Space Debris Risk: We estimate the economic risk of orbital debris by assuming 0.05 long-term fragments per launch with a cleanup valuation of 100  per fragment.

.

- Local Ecological Impact: To quantify biodiversity disruption caused by noise and chemical deposition, we introduce a sensitivity factor  into the model as

, where high-sensitivity zones like Florida wetlands are assigned .

- Model Design

Phase I: Deterministic Logistics Model Design & Economic Assessment

5.1.1  Definition of Decision Variables & Construction of Multi-Objective Function

To formulate the model, we define the following decision variables:

: Transport volume via the Space Elevator system in year . (tons)

: Transport volume via the chemical rocket system from launch site  in year . (tons)

: Binary indicator variables representing whether the transport project is active in year .

The construction of the lunar colony involves a fundamental trade-off between the total construction duration  and the total cost .

Due to the disparate dimensions of the objectives (years vs. $T), we employ the Weighted Sum Method to normalize and aggregate them into a unified objective function:

where:

is the weighting coefficient, representing the agency's preference between budget conservation and time-saving.  is a scaling factor (set to  in this study), used to align the orders of magnitude between cost and time.

5.1.2  Scenario-based Transportation Planning and Modeling

(1) Definition of General Dynamic Factors Across Scenarios

To characterize the evolution of rocket transportation costs, we define the following parameters: let  denote the cost decay rate, and  represent the asymptotic lower bound factor. Let  and  denote the unit costs in year  for direct Earth-to-Moon delivery and delivery originating from the space elevator's anchor station, respectively. The cost dynamics are formulated as:

Regarding the operational capacity of terrestrial launch sites, we introduce a growth rate . Let  be the maximum annual launch frequency for the th launch site in year . The calculation is given by:

(2) Scenario A: Transportation Using Only Space Elevator

The space elevator system operates as a two-stage logistics chain:

- Stage 1: Cargo is hoisted from the Earth terminal to the Apex Anchor via cable climbers.

- Stage 2: Cargo is then dispatched from the Apex Anchor to the lunar surface using transfer rockets.

Consequently, the total cost is partitioned into two components: the hoisting cost and the orbital transfer cost. Accounting for the fuel consumption overhead (represented by the coefficient ), the total cost  is formulated as:

The model is subject to the following constraints:

(3) Scenario B: Transportation Using Only Conventional Chemical Rockets

In this scenario, cargo is deployed via chemical rockets from ten global launch sites. These sites exhibit heterogeneity in terms of both their payload capacities and their maximum sustainable annual launch frequencies (cadence). The total cost is calculated as follows:

The model is subject to the following constraints:

(4) Scenario C: Combined Use of the Two Transport Modes

When utilizing both transportation modes concurrently, the model incorporates the full set of decision variables from both systems. The objective function for the total cost is reformulated as the superposition of the costs from Scenarios A and B:

Similarly, the mass delivery requirements are consolidated into a single global constraint:

The remaining constraints represent the union of the constraint sets defined in Scenarios A and B.

(5) Solution Approach and Preliminary Conclusions

The formulated model is characterized as a Mixed Integer Programming (MIP) problem. By leveraging the Gurobi solver to execute a Branch-and-Bound search, we ensure the identification of global optima for timeline scheduling and cost allocation, rather than settling for local optima. This provides a rigorous mathematical foundation for our subsequent strategic recommendations.

Figure 4 illustrates the optimal objective function values across different scenarios as a function of the weighting coefficient :

Figure 4: Strategy comparison under Alpha-weighted Cost Trade-offs

Deterministic results confirm that the hybrid strategy consistently outperforms single-mode alternatives. Aligned with the Theory of Comparative Advantage, this synergy leverages rockets’ early-stage time-efficiency for high-priority cargo and the space elevator’s long-term cost-effectiveness for bulk logistics. Consequently, this hybrid framework provides the baseline for all subsequent model extensions.

5.1.3  Basical Model Result

The total cost and completion time for our hybrid strategy across a range of  values are illustrated in Figure 5:

Figure 5: -Driven Time–Cost Trade-offs

Figure 5 reveals a clear Pareto frontier, demonstrating the sensitivity of project outcomes to the weighting coefficient.

To further investigate the operational details, we examine the scenario where . This value represents a balanced preference, assigning equal importance to time reduction and budget conservation. Under this specific optimization target, the temporal evolution of costs, transport volumes, and the geospatial allocation of tasks across launch sites are presented below:

Figure 6: Temporal Logistics Distribution and Geospatial Deployment

Under this balanced configuration:

- Temporal Evolution: A strategic transition is observed from early-phase space elevator dominance—driven by initial rocket costs—to rocket-centric bulk logistics, leveraging superior time efficiency as the project scales.

- Geospatial Allocation: Site utilization reflects heterogeneous capacities; the optimal solution prioritizes low-latitude launch bases, allocating significantly higher aggregate throughput to sites with superior launch frequencies.

Phase II: Stochastic Resilience & Adaptive Strategy

While Phase I yields a static mathematical optimum, its 'perfect world' assumption creates a tightly coupled system lacking redundancy.

Real-world Earth-Moon logistics face severe stochasticity, where minor faults risk cascading failures. Thus, we must shift from cost minimization to system robustness, transitioning to Phase II: Risk Modeling.

5.2.1  Mathematical Representation of Risk Features

We abstract system risks into two representative modes of supply chain disruption:

(1) Asset-Destruction Risk (Rocket Segment): Rocket failures, modeled as Bernoulli trials, incur asset and sunk costs, causing effective transport volume  to fall short of planned volume . Given a failure probability , the actual launch count follows a Binomial distribution:

Accordingly, we define the cargo loss volume  and sunk cost incurred by rocket  failures in year  as:

(2) Service Degradation Risk (Elevator Segment): Unlike binary rocket failures, space elevator malfunctions (e.g., wear or tether oscillation) manifest as Capacity Derating. We introduce a time-variant Effective Capacity Factor to quantify operational status, where induces cargo backlogs and schedule slippage. Its PMF is defined as:

Here,  denotes mechanical failures reducing annual capacity to zero, while  represents tether oscillation (induced by Coriolis forces or debris) scaling capacity by . Consequently, the revised maximum capacity ceiling in year is:

These definitions translate qualitative risk descriptions into random variables operable within our mathematical framework. This formulation encompasses both cargo loss and schedule delay risks, establishing a robust foundation for subsequent Monte Carlo stress testing.

5.2.2  Monte Carlo Stress Testing and Model Vulnerability Analysis

To quantify the operational reliability of the Phase I strategy, we developed a Monte Carlo simulation engine. Through  independent iterations, we simulated system performance under stochastic perturbations. The results are illustrated below:

Figure 7: Distribution of Actual Deliveries

Simulation results expose the deterministic model's fragility; stochastic failures plummet the success rate to 0% with a 4.68 Mt shortfall. This failure originates from a zero-redundancy design where saturated capacity utilization eliminates critical safety margins against perturbations.

5.2.3  Dual-Buffer Robust Optimization Strategy

Drawing from Resilient Supply Chain Management, we propose a Dual Buffering Strategy, trading proactive redundancy costs for system certainty.

(1) Demand-Side: Safety Stock Buffer: To address irreversible rocket-induced cargo loss, a safety redundancy coefficient  is introduced to inflate transport targets. Analogous to inventory safety stock, this buffer offsets in-transit losses, revising the demand constraint as:

This implies that the surplus transport volume is designated to offset potential attrition caused by rocket failures.

(2) Supply-Side: Capacity Discount Factor: To account for the space elevator's operational instability, we preemptively 'discount' its nominal capacity in the planning phase. We introduce derating coefficients  and , defined as:

This compels the planning algorithm to refrain from saturating nominal capacity, thereby reserving a 'Capacity Buffer' to accommodate unscheduled maintenance or reduced-speed operations. Accordingly, the revised capacity constraint is formulated as:

5.2.4  Robustness Validation and Risk Premium

Incorporating the aforementioned probabilistic constraints, we re-solved the model and executed a subsequent Monte Carlo simulation. The results indicate:

Figure 8: Comparative Analysis: Deliveries, Cost-Time Trade-offs & Cumulative Payload Loss

- Success Rate Resurgence: The system's mission success rate across all simulated failure permutations rebounded to 100%.

- Quantification of Risk Premium: This reliability entails a cost. Total expenditure escalated from $68.3 Trillion to $86.3 Trillion (+26.4%). This incremental $18 trillion should not be viewed as waste, but rather as a necessary Insurance Premium to safeguard this megaproject against catastrophic collapse.

5.2.5  Adaptive Substitution Mechanism under Extreme Scenarios

To examine the dynamic adaptation logic of the Earth-Moon logistics system under extreme conditions, we formulated four representative stress scenarios. Holding the subjective decision preference constant at , we contrasted the Transport Composition across distinct risk profiles. This analysis elucidates the adaptive substitution mechanisms intrinsic to the logistics network:

Figure 9: Multi-Scenario Evaluation of Space Elevator and Rocket Fleet Strategies

Figure 9 reveals three critical shifts in system behavior under varying risk profiles:

1. Risk Aversion under High Rocket Failure

When rocket failure rates escalate to 10%, the scheduling logic prioritizes high-certainty delivery over time sensitivity. The system fully exploits the space elevator's capacity to circumvent payload loss, deliberately extending the total project make span to hedge against the prohibitive risks of rocketry.

2. Forced Aggression under High Sway Risk

Severe tether sway represents the most challenging scenario, dropping elevator utilization to 28%. To bridge the resulting "temporal void," the model pivots from cost-efficiency to schedule-preservation, drastically increasing rocket frequency. This "forced structural drift" ensures mission completion but inevitably precipitates a significant surge in expenditure.

3. Asymmetry of Substitution Effect

A critical finding is the asymmetry between modes. Rocket substitution is "unbounded" (financially scalable), whereas elevator substitution is "bounded" (physically capped at 537,000 tons/year). Consequently, enhancing elevator stability takes strategic precedence over rocket reserves. The "Extreme Risk" scenario incurs a 7.6% cost premium ($70.9T to $76.3T), quantifying the price of logistical resilience.

This scenario-based analysis demonstrates the model's sensitivity to minute environmental shifts and its ability to generate logically self-consistent resource reorganization schemes, providing a scientific basis endowed with Strategic Flexibility for lunar base construction.

Phase III: Long-term Metabolic Supply Chain & Inventory Management

While Phases I and II concentrated on the capital-intensive demands of Construction Logistics, Phase III transitions to Operational Logistics, supporting a mature settlement of 100,000 inhabitants. This shift marks a fundamental evolution from a Project Management paradigm—prioritizing makes span minimization—to an operations management framework, designed to minimize OPEX while maximizing service levels.

5.3.1  Analysis of Annual Total Water Demand for a Lunar Colony

To achieve 'Systemic Equilibrium' in a mature lunar metropolis, annual replenishment must offset irreducible losses across three sectors: Domestic , Agricultural , and Industrial .

Based on FAO/World Bank data and NASA’ s ECLSS 99% recycling efficiency, domestic demand for a population  is：.

Agricultural demand in the Controlled Ecological Life Support System (CELSS) stems from condensation recovery inefficiencies and 'biological lock-in' (water assimilated into biomass). The model is defined as:

.

The total annual agricultural water replenishment is calculated based on the total planting area , (based on standards from NASA) the daily plant transpiration rate , the condensation recovery efficiency , and the biological lock-in volume  derived from dietary structures.

Industrial makeup water offsets electrolysis-based oxygen generation, thermal control leakage, and maintenance losses: .

Aggregating these sectors, the total annual water replenishment requirement for a lunar colony of 100,000 inhabitants  is determined to be 116,460 tons/year.

5.3.2  Supply Channel Selection and Economic Evaluation

During the operational phase, cargo—predominantly water—is characterized by its bulk volume, high predictability, and low time-sensitivity. We conducted a Comparative Cost Analysis to identify the most economically viable transport mode.

We contrasted the Annual Operating Expenses (OPEX) of a rocket-only resupply strategy against a space elevator-based approach:

With marginal costs at just 0.027% of rocketry, the space elevator is the only economically viable life-support pathway. Consequently, our model designates it as the primary routine 'Pipeline,' relegating rockets to an emergency 'Strategic Reserve.'

5.3.3  Inventory Dynamics & Schedule Optimization

To operationalize the supply chain, we applied Inventory Control Theory. We constructed a discrete-time inventory model to determine the optimal Delivery Schedule and safety stock levels. The framework is established as follows:

(1) Inventory Balance Equation

Let  denote the lunar water inventory level at the end of month . The replenishment dynamics are governed by:

Where  denotes the total volume of water arriving on the Moon in month , and $c$ represents the constant monthly water consumption.

(2) Safety Buffer Constraints

Integrating the risk assessment from Phase II, we introduce Service Level Constraints. Given that Tether Sway could precipitate a service interruption of approximately one month, we established the following boundary conditions :

- Safety Stock Threshold: . This redundancy guarantees that even in the event of a 30-day elevator outage, the colony can sustain its fundamental metabolic requirements.

- Storage Capacity Upper Limit: The maximum physical capacity of the lunar water storage facility is constrained to .

5.3.4  Optimized Resupply Schedule and Strategic Analysis

Leveraging Gurobi optimization, we formulated a comprehensive annual 'Logistics Schedule' for the lunar colony. To achieve load smoothing across the Earth-Moon logistics network, we propose the 'Weekly Pulse Strategy'.

1. Optimal Schedule

- Replenishment Frequency: Maintained as a routine weekly docking event.

- Single Mission Payload: Approximately 2,240 tons per shipment (equating to a monthly delivery volume of 8,960 - 11,200 tons).

This replenishment intensity utilizes merely 21.6% of the space elevator's total monthly capacity (based on an annual throughput of 537,000 tons). The remaining capacity can robustly support the daily passenger transit and industrial expansion required for a population of 100,000.

2. Visualization Analysis: Saw-tooth Dynamics

As illustrated in Figure 10, the lunar base's inventory level exhibits a classic Saw-tooth Pattern.  At each replenishment node, the inventory level surges instantaneously upon cargo arrival, followed by a gradual linear decline due to the metabolic consumption of the inhabitants. Because the SSRIM model incorporates the risk buffers defined in Phase II, the inventory trajectory consistently operates above the safety red line of  tons.

Figure 10: Dynamic Water Inventory Management with Resupply and Cost Over Time

3. Strategic Insights: Long-term Operations Assessment

The analysis demonstrates that leveraging the LUNALINK system keeps the cost of maintaining the 'respiration and metabolism' of a 100,000-person city (at $155 million annually) negligible and controllable within the lunar colony's economic structure. This characteristic of 'High CAPEX, Low OPEX' further validates the strategic value of the space elevator as the fundamental artery between Earth and the Moon.

Phase IV: Environmental Exteriority & Green Evaluation

With the logistics network established, we proceed to address the 'hidden costs' of space exploration. Our objective is to internalize negative externalities—specifically pollution and debris—into the decision-making framework, thereby catalyzing a paradigm shift from 'Cost Minimization' to 'ESG (Environmental, Social, and Governance) Optimization'.

5.4.1  3D Multi-objective Optimization Strategy

To quantify the environmental impact, we defined four baseline indices per launch: Atmospheric Damage , Resource Depletion , Risk Equivalent  and Local Ecological Impact . The numerical distribution of these indices is presented below:

Figure 11: Environmental Impact: Four Baseline Indices with Distribution

We integrated the Environmental Impact Index (EII) into the objective function. To maintain decision neutrality and ensure consistency and comparability with previous phases, we assumed equal importance for Time efficiency and Economic Cost (), thereby concentrating variability solely on the environmental weight, . Consequently, the new objective function is redefined as:

The constraint  isolates the environmental variable, enabling us to clearly observe the system's structural realignment of transport resources as  increases.

5.4.2  Analysis of Environmental Trade-offs

We conducted a sensitivity analysis on  to delineate the Pareto Frontier of the Earth-Moon logistics system. The results are illustrated in the figure below:

Figure 12: Time-Cost Pareto Frontier with Environmental Priority Sensitivity

Sensitivity analysis of the environmental weight  reveals a Pareto Frontier defining the trade-off between high-cost, rapid rocket-based deployment  and low-cost, eco-friendly space elevator logistics .

The system exhibits a critical strategic fulcrum at . Below this threshold, heavy-lift rockets maintain construction intensity; above it, the space elevator achieves dominance. Paradoxically, the "Eco-friendly Scenario" proves more financially economical in the long run due to the elevator’s superior unit-cost efficiency. While the Environmental Impact Index (EII) drops precipitously for , further increases lead to diminishing returns and prohibitive temporal costs—exceeding 220 years for .

Strategic Recommendation: We propose an optimal decision interval of . This range suppresses environmental impact to below 40% of the baseline while maintaining a balanced construction cycle (~140-150 years).

Ultimately, the LUNALINK system functions as a self-regulating framework, enabling precise calibration of "greenness" to realize Sustainable Interstellar Civilization Governance.

6 Sensitivity Analysis

- 6.1 Global Parameter Screening: The Morris Method Approach

To navigate the high-dimensional uncertainties of lunar colonization, we employ the Morris Elementary Effects Method to screen parameters based on their influence magnitude  and degree of non-linear interaction .

Figure 13: Multi-Dimensional Pareto Analysis of Time, Cost, and Environmental Impact

Key findings across the objective dimensions include:

- Cost & Decision-Making: Rocket pricing and the policy-weighting factor  emerge as the most profound drivers. Their location in the high-quadrant indicates that project costs and payload allocation strategies are governed by complex, non-linear interactions between these factors.

- Time Sensitivity: The cost decay rate is the dominant determinant for project duration. This highlights technological maturation—manifested as accelerated cost reduction—as the primary catalyst for schedule compression.

- Environmental Impact: A market-driven feedback loop is observed; lower rocket costs incentivize intensive deployment, thereby exacerbating atmospheric degradation and energy consumption. Conversely, higher costs act as an implicit constraint that promotes environmental sustainability.

In summary,  and  serve as the fundamental anchors for system optimization and strategic planning.

- 6.2 Quantitative Local Sensitivity Analysis

Building on the Morris method, a One-at-a-Time perturbation () was employed to quantify the local impacts of key parameters. The resulting Tornado Diagrams (Figure 14) rank the influence of these variables on timeline, cost, environmental footprint, and transportation structure.

Figure 14: Sensitivity and Trade-off Analysis of Time, Cost, and Env Impact

（1）Impact of Technological Iteration on Construction Timeline

Project duration is critically sensitive to the Tech Decay Rate . Avariance in  yields a 53-year swing (117–170 years) relative to the 131-year baseline. This "Technology Dependency Trap" underscores that, under fixed capital investment, the iteration speed of rocket cost-efficiency constitutes the primary scheduling bottleneck.

（2）Rocket-Cost-Driven Budgetary Framework

Rocket Unit Cost  emerges as the dominant budgetary driver. A 20% cost reduction precipitates a 60% budget decrease (from $70.14T to $28.5T), while a 20% overrun elevates it to $83.6T. This high leverage stems from the heavy front-loaded reliance on chemical rockets, making launch cost mitigation (e.g., via reusability) a financial imperative.

（3）Reliability-Driven Environmental Footprint

Counter-intuitively, the Rocket Failure Rate  is a more significant environmental determinant than the weighting coefficient . A  fluctuation in shifts the Environmental Impact Index between 110 and 140 M Pts. This validates a "Safety is Green" paradigm, where enhancing system reliability is more effective for ecological protection than policy-based penalties.

（4）Highly Sensitive Transport Structure

The parameter () governs the system's "morphology." A 20% improvement in increases the space elevator’s cargo share from 65% to 85%; conversely, underperformance drops its share to 40%. This high "Structural Elasticity" allows the LUNALINK project to pivot between rocket-centric and elevator-centric pathways based on technological maturity.

7 Strengths and Weaknesses

- 7.1 Strengths

- Our LUNALINK strategy provides a holistic logistics roadmap, spanning from initial infrastructure construction to long-term metabolic maintenance (water resupply). It bridges the gap between macro-level transportation planning and micro-level inventory management.

- Our strategy is risk-awareness. By simulating stochastic failures—such as rocket launch anomalies and space elevator oscillations—through Monte Carlo stress testing, it’s proved that our model can still provide valuable strategies under the impact of certain external risks..

- Our model internalizes environmental externalities (e.g., atmospheric damage, space debris) into the objective function. This allows for a 3D multi-objective optimization that balances economic cost, time efficiency, and "Green" interstellar governance.

- The "Weekly Pulse Strategy" offers a concrete, discretized timetable for water delivery. By utilizing inventory control theory, the model guarantees the sufficient water of 100,000 inhabitants even during extended elevator outages, making the plan practically viable.

- 7.2 Weaknesses

- Our model relies on the extrapolation of current technological trends to the year 2050 and beyond. Factors such as the cost decay rate and  are based on theoretical assumptions that may be subject to unpredictable paradigm shifts in aerospace engineering. But this is unavoidable when predicting future scenarios.

- We simplified the orbital dynamics. To maintain computational feasibility over a century-long horizon, the model utilizes linearized approximations for complex orbital transfer scenarios and fuel consumption ratios. This may omit higher-order perturbations in Earth-Moon logistics.

8 Conclusion

In this study, we developed the LUNALINK framework, a systemic solution for establishing and sustaining a 100,000-person lunar colony. Our analysis demonstrates that a single-mode transportation system is insufficient for the demands of lunar colonization. Instead, a Rocket-Space Elevator Hybrid Strategy provides the optimal balance, leveraging the immediacy of rockets for early-stage construction and the cost-efficiency of elevators for bulk operational logistics.

Key findings indicate that while risk-blind deterministic plans suffer from a 0% success rate under stochastic failures, our Dual-Buffer approach restores reliability to 100% by introducing strategic safety stocks and capacity derating. Furthermore, for long-term operations, the space elevator proves to be the only financially viable "artery," reducing water resupply costs by over 99.9% compared to traditional rocketry.

Finally, by applying the Economic-Efficiency-Green-Evaluation model, we identified a Pareto optimal decision interval for environmental weighting. This suggests that the transition to a sustainable interstellar civilization requires a deliberate shift from pure cost minimization to a balanced governance model that respects the terrestrial and lunar ecological footprints. The LUNALINK framework provides a robust, quantifiable, and environmentally responsible blueprint for humanity’s expansion into the cosmos.

MEMORANDUM

To: The Director of the Moon Colony Management (MCM) Organization

From: Team 2601727

Date: January 28, 2026

Subject: Strategic Roadmap for Establishing and Sustaining a 100,000-Person Lunar Colony

Distinguished Members,

Based on our simulation of the Earth-Moon Logistics Network, we propose the "LUNALINK" strategy. Our research indicates that only a systemic approach integrating "Hybrid Transportation, Resilient Redundancy, and Green Sustainability" can ensure absolute mission success and economic feasibility over a construction horizon spanning more than a century.

I. Construction: Hybrid Drive & "Dual-Buffer" Resilience

During the infrastructure construction phase, we strongly recommend abandoning a single-mode transportation model in favor of a Rocket-Space Elevator Hybrid Strategy, leveraging rocket immediacy for early phases and elevator cost-efficiency for bulk transport. Static plans yield a 0% success rate under risk; thus, we propose a "Dual-Buffer Robust Strategy" featuring demand-side safety stocks and supply-side capacity discounts.

- Demand-Side: Introduction of a safety stock buffer to offset potential asset losses resulting from rocket launch failures.

- Supply-Side: Application of a capacity discount factor during planning to accommodate service degradation caused by tether sway in the space elevator system.

While this raises the budget to $86.3 trillion (a 26.4% premium) and extends the duration to 147 years, it restores success rates to 100%. An adaptive mechanism is also required to dynamically switch modes during failure events.

II. Operations: Metabolic Balance & Pulse Resupply

Sustaining the colony requires 116,460 tons of water annually. Rocket resupply would cost $573.8 billion/year, whereas the space elevator costs only $0.155 billion—making it the only viable pillar. We recommend a "Weekly Pulse Strategy" (~2,240 tons/week) to maintain inventory above the 14,558-ton safety line, guaranteeing survival during 45-day elevator outages.

III. Environmental Responsibility

In the pursuit of interstellar expansion, we must not ignore the negative externalities imposed on the terrestrial environment. Using our EEGE model, we advise setting the environmental weight coefficient  to [0.35, 0.45]. This interval balances a feasible 140–150-year cycle with a >60% reduction in pollution and space debris risks. This represents the "Pareto Optimal" path between engineering efficiency and ecological ethics.

In conclusion, the construction of a lunar colony should not merely be a race for speed, but a long-term investment in system robustness and sustainability. The LUNALINK framework provides a quantifiable, risk-resistant, and environmentally responsible blueprint for implementation.

References

- SpaceX, "Falcon Heavy," [Online]. Available: https://www.spacex.com/vehicles/falcon-heavy. [Accessed: Feb. 2, 2026].

- Our World in Data, "Cost of space launches to low Earth orbit," [Online]. Available: https://ourworldindata.org/grapher/cost-space-launches-low-earth-orbit?time=2013.latest. [Accessed: Feb. 2, 2026].

- FAO, "FAO Data," [Online]. Available: https://data.apps.fao.org. [Accessed: Feb. 2, 2026].

- World Bank, "World Bank Open Data," [Online]. Available: https://data.worldbank.org. [Accessed: Feb. 2, 2026].

- NASA, "International Space Station Environmental Control and Life Support System (Fact Sheet)," [Online]. Available: https://www.nasa.gov/wp-content/uploads/2015/05/60484main_fs_eclss.pdf. [Accessed: Feb. 2, 2026].

- Lunar Palace 1 Project Team, "Lunar Palace 1: A ground-based experiment for long-duration human space missions," [Online]. Available: http://lunarpalace.buaa.edu.cn. [Accessed: Feb. 2, 2026].

- NASA, "Advanced Life Support Baseline Values and Assumptions Document (BVAD)," [Online]. Available: https://lsda.jsc.nasa.gov. [Accessed: Feb. 2, 2026].

Report on Use of AI Tools

- Gemini 3 Pro (Jan 31, 2026 and Feb 1, 2026 version)

Purpose: Data Collection

Query: "Please collect relevant data on Falcon rockets and provide URLs; collect global water usage statistics and provide relevant data sources."

Output: The model provided a list of web URLs and descriptions of potential data sources regarding SpaceX Falcon rocket specifications and global water consumption trends. We verified the accuracy of these links manually and downloaded the verified data for further analysis.

- Gemini 3 Pro (Feb 2, 2026 version)

Purpose: Translation and Language Polishing

Usage: Utilized these tools to translate our original draft written in Chinese into English and to refine the grammatical structure of the final report.

- ChatGPT 4o (Feb 2, 2026 version)

Purpose: Code Debugging and Visualization Assistance

Query: "I am using Python's Matplotlib to draw a 3D scatter plot representing the Pareto Frontier of Time vs. Cost vs. Environment. My code keeps returning a 'dimension mismatch' error. How can I fix this and make the plot more visually appealing?"

Output: The model identified an error in the array shape of our Z-axis data and provided a corrected code snippet. It also suggested using a color map (cmap='viridis') to better distinguish the environmental impact levels, which we implemented in Figure 13.

- Claude 3.5 Sonnet (Feb 1, 2026 version)

Purpose: LaTeX Equation Formatting

Query: "Convert the following mathematical relationship into standard LaTeX format: The rocket transport cost equals the base cost times the exponential decay function of time, considering a lower bound factor."

Output: The model generated the LaTeX code for our cost dynamic equation (Equation 0.2). We manually checked the symbols to ensure they matched our Notations table before inserting it into the paper.